Holden Colorado 2012-2020
Load-Lid Central Locking Fitting Instructions

Thank you for purchasing a quality Utemaster™ product, our aim is providing you with as much assistance as possible throughout the fitting process.  Please contact us if assistance is needed.

We recommend watching the installation video before attempting to install

Enter this address into your web browser to view the fitting video:
https://wi.st/2HJ58qs

DESPATCH:

1x 80606-17 / WHUTELL-COL 1.1

Step 1) Connect the relays

Step 2) Cable tie the relays in place

Step 3) Remove the earth bolt

Step 4) Connect the earth cable to the earth point and re-install the bolt

Step 5) Open the battery cover and remove the nut shown

Step 6) Attach the power cable to the thread and re-install the nut. Close the battery cover

Step 7) Remove the plastic caps and rubber seal to expose the wiring behind the kick panel

Step 8) Locate the cab entry point from the interior

Step 9) Make a small incision in the cab entry rubber seal

Step 10) Insert a wire through the incision

Step 11) Tape the Lock/Unlock cable to the wire and draw it back through into the cab interior

Step 12) Behind the kick panel, undo the plug to make it easier to access the wires

Step 13) Identify the wires with the colour blends indicated

Step 14) Place one of the provided clips over the Grey wire

Step 15) Insert the labelled ‘Lock’ into the plastic clip alongside the Grey/Brown wire

Step 16) Clamp the clip closed until the metal plate is no longer protruding, and close the clip

Step 17) Repeat Steps 14-16 for the Brown/Yellow wire

Step 18) Insert the plug back into its socket

Step 19) Re-install the plastic caps & rubber seal in the passenger’s side door

Step 20) Take the last cable remaining on the loom and feed it down the firewall so it comes out underneath the vehicle

Step 21) Run down the length of the chassis and cable tie in place

Step 22) Pull up through the gap between the cab and wellside

Step 23) Install the Load-Lid as per fitting instructions provided

Step 24) Connect the vehicle wiring loom to the plug already installed in the Load-Lid

Step 25) Cable tie any slack cables safely out of sight and where they won’t hinder moving parts

Your central locking is now fully fitted and ready to use.

Thank you again for using Utemaster™ and don’t forget to keep checking our website for our latest innovations!